INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

S.C.


[image: image1.wmf]  S.A.
__________________________________________________________________________________________________________________________
	MPRCL

Roll mill cylinders grinding portable machine 


[image: image2.png]



	The machine has in componance a grinding head with hydrodinamic bearings driven by an electric motor of 3 kW.
The longitudinal movement is ensured by an d.c. electric motor  and a ball screw-nut mechanism.

The transversal movement is ensured by a mechanic copy mechanism witch ensure the demanding shape for cylinders.




TECHNOLOGIC  CAPABILITIES
	The roll mill cylinders grinding portable machine ensure the debug of the cylinders without the dismantle, the machine is mounted on the roll mill stand. It can be realised cylindrical, concave or conical shapes.



TECHNICAL  FEATURES
	- Grinding max diametre




mm

550

- Grinding max lenght





mm

1300

- Swellness or concavity




mm

0 – 3

- Max inclination





grd.

10

- Grinding whell speed




rot/min

750 si 1500

- Longitudinal feed speed




mm/min
50 – 500

- Machining accuracy







h7

- Machined surface roughness



μm

1,6 




	INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -
ROMANIA - 030352, Bucharest 49, Bvd. Basarabia  250

Tel/ Fax:: (+4021) 255 17 70, mobile phone +40745 581 518

E - mail: office@simtex-mu.ro    http://www.simtex-mu.ro



[image: image3.wmf]
_1169423357

_1169423310

