INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

S.C.


[image: image1.wmf]  S.A.
__________________________________________________________________________________________________________________________
	MSD-02
Dressing special machine



[image: image2.png]



	The special machine is composed of:

- basic machine

- manipulator

- dressing transportation system

- part tightening device

- axial part positioning system

- tools placement

- lubricating installation

- pneumatic installation

- electrical installation

The machine works on base of the principle: fixed part, rotating tool.




TECHNOLOGIC POSIBILITIES
	The machine performs technical operations of exterior cylindrical turning, from center to exterior for the two ends of the part.


TECHNICAL FEATURES
	- Dimensions of the part to be machined: - diameter
     mm

       13,08 

 - length

     mm

       492,89 

- Working cycle





     -

       semiautomatic
- Dressing machine time




     sec

       24 

- Manipulator time





     sec

       22,5 

- Productivity 80%





     pcs/h
       120 

- Capacity of the transporter, max.

· loading post





     pcs.
       50

· unloading post





     pcs

       50
- Transporter charging, max.




     daN /m.l
       20
- Transportation feed





     m/min
       1,2 

- Arm stroke basic machine, max.



     mm

       2 

- Tailstock stroke basic machine, max.


     mm 
       30

Manipulator:

· Working stroke: - X axis




     mm

       1250

-  Z axis




     mm

       200 

- A axis




     grad
       60

- part catching mechanism

     mm

       10 

· Displacement feed:  - X axis



     m/min 
       24

 - Z axis



     m/min
       18 

 - A axis



     grad/sec
       60

 - part catching mechanism

     m/min 
       4 

- Tailstock speed (variable)




     rot /min
       600 ÷ 1000 

- Electric installed power




     kW

       2,2 

- Air pressure






     bar

       4
- Overall dimensions (L x W x H)



     mm

       3759x2415,5x1900
- Net weight






     kg

       1580 


MACHINING PIECE (CANDU 6 FUEL ELEMENT)


[image: image3.wmf]1.6

1.6

=

=

max. 9.27

29.5

=

min. 492.89

=

194.8

194.8

=

29.5

=

Ø13.08 

±0.05

Ø13.18 ± 0.01

max. Ø13.23

(4.55)

=

max. 493.15

min. 1.32

Ø13.18 ± 0.01

max. Ø13.23

max. 1.39

(4.55)

=


	INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -
ROMANIA - 030352, Bucharest 49, Bvd. Basarabia  250

Tel/ Fax:: (+4021) 255 17 70, mobile phone +40745 581 518

E - mail: office@simtex-mu.ro    http://www.simtex-mu.ro



[image: image4.wmf]
_1169423357

_1203936580.dwg

_1169423310

