INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

S.C.


[image: image1.wmf]  S.A.
__________________________________________________________________________________________________________________________
	MSDT-02
Plug - scabbard welding machine for nuclear industry



[image: image2.png]



	The special machine is composed of:

- bed

- scabbard welding head

- scabbard displacing carriage

- scabbard tightening cartridge

- displacing and scabbard sliding perceiving transducer

- scabbard purging system with helium

- plugs loading system

- scabbards loading and cleaning by inhalation system

- pneumatic installation

- electrical installation




TECHNOLOGIC POSIBILITIES

	The machine performs the technical operation of welding the plugs on the scabbards containing nuclear fuel used in CANDU - type electric plants, through electric resistance in helium environment.




TECHNICAL FEATURES

	- Dimensions of the part to be welded:
· scabbards: - diameter




mm

     13,08 ± 0,05
- length




mm

     485,7 ± 0,1 

· plug: - diameter





mm

     13,31 ± 0,03
  - length 





mm 

     4,55 ± 0,03 
- Working cycle





-

     semiautomatic
- Positioning precision:
· concentricity





mm

     ± 0,05
- Productivity






scabbards/h
     105 

- Electrode pressing force, max.



daN

     100 

- Electric installed power




kVA

     ~ 42 

- Air working pressure




daN/cm2
     4 ÷ 6 

- Helium pressure, min.




daN/cm2
     6,9 

- Welding current strength




kA

     8 ÷ 12 

- Overall dimensions (L x W x H)



mm

     2280x1080x1650

- Net weight






kg

     900 




MACHINING PIECE (CANDU 6 FUEL ELEMENT)


[image: image3.wmf]max. 9.27

194.8

=

min. 492.89

29.5

(4.55)

Ø13.08 

±0.05

194.8

=

=

 Ø13.31

=

=

=

max. 493.15

min. 1.32

29.5

max. 1.39

(4.55)

=

=

 Ø13.31


	INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -
ROMANIA - 030352, Bucharest 49, Bvd. Basarabia  250

Tel/ Fax:: (+4021) 255 17 70, mobile phone +40745 581 518

E - mail: office@simtex-mu.ro    http://www.simtex-mu.ro



[image: image4.wmf]
_1169423357

_1204529951.dwg

_1169423310

