INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

S.C.


[image: image1.wmf]  S.A.
__________________________________________________________________________________________________________________________
	SE 450 x 1250 R
Shaft turning module portable machine 


[image: image2.png]



	SE 450 x 1250 R shaft turning modulated portable machine is designed to correct shafts cylindrical surfaces, mounted in bearings without their dismounting or their travel at a specialised unit. Its mechanisms perform the necessary kinematics of shafts machining.




TECHNOLOGIC  CAPABILITIES
	The working principle and its engineering elements. The main machines elements are: two fixed centering and fixing rings; three cylindrical bares on which motion the axial work support; work support performing two motions (rotation, translation); a feed box with 4 stages; a leading screw. The drive is done by means a hydraulical engine which rotates the crown gear on which is assembled the cutter-holder supports. The feed motion is taken over the crown gear by the feed box, passing to the leading screw the necessary feed range.



TECHNICAL  FEATURES
	- Min/Max worked shaft diameter



mm


200/450

- Max worked lenght





mm


1250

- Machining accuracy





-


h7

- Max rotate speed





rot/min


30

- Automate longitudinal feed (4 stages)


mm/rot


0.1 – 0.4

- Cross feed






-


manually

- Machined surface quality (Ra)



μm


3.2

- Installed power





kW


7





	INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

ROMANIA - 030352, Bucharest 49, Bvd. Basarabia  250

Tel/ Fax:: (+4021) 255 17 70, mobile phone +40745 581 518

E - mail: office@simtex-mu.ro    http://www.simtex-mu.ro



[image: image3.wmf]
_1169423357

_1169423310

